Royal Observatory, Edinburgh

Blackford Hill

Edinburgh, EH9 3HJ

United Kingdom

Telephone:	+44 (0)131	668 8100

Facsimile:		668 8264 (Central)

662 1668 (West Wing)

668 1130 (Villas)

662 1654 (Directorate)

Email:	initial.surname@roe.ac.uk

WWW: 	http://www.roe.ac.uk/

ICD 1.0 version for NAOMI WFS Camera

Part two- VME interface

Xiaofeng Gao

11 January, 2000

Contents list

1. Introduction	3

2. References	3

3. The control flowchart	3

4. Readout mode [2]	4

5. Hardware configuration  for synchronized 2-CCD readout	4

6. SDSU VME I/F parallel port and C40 link convert (SDSU-RS422) card interface	5

7. The data protocol for NAOMI WFS camera	6

8. The FSTART and FIFORD waveform	7

9. Error handling	8

10. Commands and replies	9

11.  VME I/F board DSP codes for NAOMI WFS camera	10

12. Modification to SDSU VME I/F board	11

13. CCD readout procedure ( for engineering level)	13

14. Parameters  and memory map for VME I/F board DSP	14

15. Check for code ID, author and version	15

16. Communications between SDSU controller Timing board and VME I/F board	15

17. Appendix	16

ICD 1.0 version for NAOMI WFS Camera

Part two- VME interface

# 1. Introduction

This note describes the proposed control flowchart, frame data protocol and commands/replies between host, SDSU VME I/F cards and controller, VME I/F cards and C40 for NAOMI WFS camera.  A brief description of  communication link between/in SDSU Gen II controller and VME  is also included as a background information for understanding the SDSU controller.

# 2. References

[1] “System description”, SDSU document

[2] “ CCD readout mode”, Derek Ives, UK.ATC

[3] “Timing board”, "VME Interface board",  SDSU document Updated MAY,25 1998

[4]“ CCD controller requirements for ground-base optical astronomy”  Robert Leach

[5] “ ICD1.6/1.10 Revision Working Document – version 7”, Gemini document

# 3. The control flowchart

The NAOMI WFS camera system has two CCD headers with two SDSU CCD controllers and two VME I/F cards. They are called master and slave.  Each SDSU VME I/F board sits at its own address.  The central control system is called Electra ( built in Durham ).  The image data is fed from the CCD cameras directly to the ELECTRA system, by-passing the VME bus in the WFS VME control System.  The ELECTRA system then performs the image centroiding calculations.   Commands are issued from host ( ELECTRA),  through  Epics/vxWork to  call a function from sdsuLib, which communicates with VME I/F board through VMEbus, using a SDSU-defined protocol ( header [source board, destination board , No of words], command and No of arguments if any).  The DSP on the VME I/F board checks the in-coming command, it then either goes to a particular subroutine to execute the command, or pass the command to the Timing board (inside controller), which also checks the command and either goes to a particular subroutine to execute the command.  When a command is completed, each board (destination board)  usually sends a reply back to its source board.  The control flowchart is shown in Figure 1.

MVME167

Electra

Interface

Epics / vxWorks

SDSULib

SDSU VME I/F  card

SDSU CCD controller

C40 convertor card

C40 Interface

C40 Array

SDSU VME I/F  card

SDSU CCD controller

C40 convertor card

C40 Interface

Commands

data

data

Figure 1.  The control flowchart

# 4. Readout mode [2]

The different readout modes for Naomi WFS camera are described in more details  [2]

# 5. Hardware configuration  for synchronized 2-CCD readout

The two controllers in Naomi WFS camera can be operated  independently, they then look like just  two separate cameras.   For synchronized readout,  the master and slave controller have to be hardware configured( Figure 2, 3).  The “SYNC” signal ( from pin HR/W) by master is hardware decoded to generate an IQRA interrupt to both  master and  slave  ( Figure 2), which will jump to interrupt subroutine once detecting the IQRA.  However, the start of synchronized 2-CCD readout  is trigged only after a second "SYNC" signal is issued by the master.  In order to maximize synchronization for each frame, when the first frame is finished, the slave will wait for a "SYNC" signal issued by the master to start the next frame with the master, and so on.

Extal

DSP

HR/W

IQRA

50MHz clk

Extal

DSP

HR/W

IQRA

###### Clck, 50ohm cable

###### SYNC 50 ohm cable

Master Timing board

###### Slaver Timing board

J16

J18

J19

J17

J216

50MHz clk

J216

50 ohm

Vc

Figure 2   the Master and Slave configuration

JP16

JP17

JP18

JP19

JP21

EEPROM

JP3  *   *   *

JP2  *   *   *

JP3  *   *   *

JP2  *==*   *

JP3  *==*   *

JP2  *==*   *

JP3  *   *==*

JP2  *   *==*

Setting for

28c256, read only

Setting for

28c256, read/write only

Setting for 27c256

29c256, read only

Master:  JP16,JP18  and JP21 in

Slave:   JP17,JP19 in, R8=50R must be in.

Figure 3   the Master and Slave jump settings on Timing board

# 6. SDSU VME I/F parallel port and C40 link convert (SDSU-RS422) card interface

The image data from CCD controller will pass SDSU VME I/F card through on board parallel port to C40 array by SDSU_RS422 interface card.  During  CCD readout, the DSP on VME I/F board will generate a “FSTART” signal (pulse) on parallel port as a frame start indication to SDSU_RS422 interface card in each frame transfer, it will also generate “RMT_RST” signal on the parallel port, which is then fed back to  “FIFORD” as FIFO readout signal to read out each coming data in FIFO from Timing board.  At the same time, this “RMT_RST” is used as a strobe signal to C40 interface card.

The functional block and link  to SDSU_RS422 card are illustrated in Figure 4

Data from

Timing board

FIFO

Low 8Bit

FIFO high

8 bit

D0

(1 ~16)

D15

EF (21)

FIFORD (19)

RMT_RST (17)

FSTART (18)

PAL

S-P shift register

VME  DSP      Data

port B.1      .           port B.8

Fiber

Receiver

Latch

VMEbus

16

Bit

Data

D24 parallel port on VME IF

SDSU-RS422 card

Figure 4. The function block and link to SDSU_RS422

The parallel connector on the SDSU VME I/F board is two row 24  IDC type,  see below for enclosed photo Figure 5.  The pin assignment of the parallel port  can be found in  SDSU document  (vmesch7A.ps)

Figure 5. the SDSU VME I/F board

# 7. The data protocol for NAOMI WFS camera

Controller to VME I/F board Frame data Format

In response to readout CCD command, the downlink (Timing board to VME interface board) is changed from 32-bit to 16-bit format. The uplink ( VME interface board to Timing board) format remains unchanged (24bit).  The Controller starts to do CCD readout task.  Once the exposure period has elapsed, 10 of 14-bit header words are transmitted, followed by a sequence of 16-bit image pixel values. The last 14 bit word in a frame data is Zero indicating the end of frame. This procedure repeats until an ABORT command from host is received.  The downlink format is then changed back to 32-bits.

The frame format is shown in Table 1. It is self-defined

Table 1 Format of Controller Data Frame

| Word Number | Name | Description |
| --- | --- | --- |
| 0 | 0000 |  |
| 1 | 0000 | The two zeros are Frame sync words |
| 2 | OPMODE | Repeated here for sync propose |
| 3 | OPMODE | the current operation mode. DEFINATION [2] |
| 4 | FRMCH | Two words for frame counter, starting from 1, |
| 5 | FRMCL | rounded back if 28 bit reached |
| 6 | EXPSH | High byte of the exposure time |
| 7 | EXPSL | low byte of the exposure time |
| 8 | ROWS | The number of rows /frame |
| 9 | COLS | The number of columns /frame |
| 1+9 | PIXEL(1,1) | First pixel value. |
| : | : | : |
| N+9 | PIXEL(4,N) | Last pixel value. |
| N+1+9 | 0000 | End of frame |

N -Number of pixels /frame

VME I/F board to C40 Frame data Format

In response to an RDS or RDC command from host, the VME interface board fiber optic shift register is changed from 32-bit to 16-bit format.  Knowing there is data about to arrive from FIFO, the DSP on VME I/F board waits, until the beginning of frame header (0000,0000) is received. It then starts to transmit 7 14- bit header words to C40, followed by a sequence of 16-bit pixel values. This procedure repeats until an ABORT command from the host . The fiber optic shift register then is changed back to 32-bits.

The frame data format sent to C40 is shown in Table 2.

Table 2 Frame data Format for VME I/F board to C40

| Word Number | Name | Description |
| --- | --- | --- |
| 0 | OPMODE | the current operation mode. DEFINATION [2] |
| 1 | FRMCH | Two words for frame counter, starting from 1, |
| 2 | FRMCL | rounded back if 28 bit reached |
| 3 | EXPSH | High byte of the exposure time |
| 4 | EXPSL | low byte of the exposure time |
| 5 | ROWS | The number of rows in frame/frame |
| 6 | COLS | The number of columns in frame./frame |
| 1+6 | PIXEL(1,1) | First pixel value. |
| 8 | : | : |
| N+7 | PIXEL(4,N) | Last pixel value. |

N -Number of pixels /frame

# 8. The FSTART and FIFORD waveform

FSTART

Header

Frame transfer

One frame pixels

RMT_RST

FIFORD

Latency

Timing

| Timing |  | Voltage level on the parallel port CON24 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | D0~D15 ( pin:1-16) | Voh=2.4v (min) | Vol=0.4v (max) | IDT7202 |
| Tr | 6 ns (min) |  | Vih=2.0v(min) | Vil=0.8v (max) |  |
| Tf | 6 ns (min) | EF (pin:21) | Voh=2.4v (min) | Vol=0.4v (max) | IDT7202 |
| Ta | 15 ns (max) | FIFORD (pin:19) | Vih=2.0v(min) | Vil=0.8v (max) | CY7C31 |
| Tdv | 5 ns (min) | RMT_RST (pin:17) | Voh=2.4v (min) | Vol=0.4v (max) | DSP |
| T1 | Exposure time | FSTART (pin:18) | Voh=2.4v (min) | Vol=0.4v (max) | DSP |
| T2 | 21.6 us (min) |  |  |  |  |
| T3 | 120 ns (min) |  |  |  |  |
| Tw1 | 40 ns (min) |  |  |  |  |
| Tw2 | 40 ns (min) |  |  |  |  |

Note: the width of FSTART and FIFORD, ie, Tw1,Tw2 can be adjusted by user in software.

The measured FSTART and FIFORD waveform

Figure 6  The measured FSTART and FIFORD waveform

# 9. Error handling

The error handling in the current system has not yet been fully defined.  It is proposed that it will depend on the frame status condition word (FSTAT), which is defined in Table 3

Table 3 Frame Status word bit definition

|  | Bit |  | Error |
| --- | --- | --- | --- |
| HD_WAIT | 0 | Set if waiting for a frame header, indicates where the DSP is, |  |
| EOF_ERR | 1 | Set if end of frame error | yes |
| FBA_VLD | 2 | Set if current frame buffer address is valid |  |
| LST_FRM | 3 | Set if last frame or time out |  |
| ABRT | 4 | Set if Abort CMD received during readout |  |
| TIM_OUT | 5 | Set if time out period 65ms has reached during FIFO readout | yes |
| FRDING | 6 | Set if frame readout is in process, *this can be changed to anything later |  |
| FBA_WAT | 7 | Set if wait for FBA, only for indication where the DSP is |  |
| HF_WAT | 8 | Set if wait for Half FIFO time out (=FO_TO) | yes |

The frame status word is only sent back to Vxwork not to C40

The VME I/F DSP will not take any action apart from reporting during error handling, at any point in reading FIFO, if there is time out, the DSP will jump out, flag Frame status word( FSTAT), send FSTAT back to Vxwork, generate a frame INT, then jump to check CMD routine.  The readout process will not stop until it is asked by host to "Abort"

# 10. Commands and replies

The  commands for SDSU VME I/F are listed in BOOT and APPLICATION tables ( all the command will have a 24 bits header associated with it, source always is host)

Table 4 SDSU VME BOOT Commands Defined for  WFS

| Command | Words include header | Valid Destination DSPs | Response | Description |
| --- | --- | --- | --- | --- |
| LDA, Nnnnnn | 3 | VME ¦ TIMING | DON | LoaD Application program #nnnnnn from EEPROM. |
| RDM, Maaaaa | 3 | VME ¦ TIMING | Dddddd | ReaD Memory. Read DSP address maaaaa. Returned data = dddddd. The most significant nibble of the address indicates the memory type. m = 1: P memory m = 2: X memory m = 4: Y memory m = 8: EEPROM |
| WRM, maaaaa, dddddd | 4 | VME ¦ TIMING | DON | Write Memory. Write dddddd to DSP address maaaaa. The most significant nibble of the address indicates the memory type. See RDM |
| TDL, Nnnnnn | 3 | VME ¦ TIMING | Nnnnnn | Test Data Link. Destination DSP echoes nnnnnn back to source. |
| DON | 2 | VME ¦ TIMING | DON | Un-used command |
| SRA, hiaddr, loaddr | 4 | VME | DON | Set Reply Address. Defines the start of a circular buffer in VME memory that receives replies from the VME interface DSP. |
| RRS | 2 | VME | SYR (fromTiming) | Remote Reset System. Reset the Timing and Utility DSPs. |
| CHK | 2 | VME | nnnnnn | Checksum VME board memory areas where nnnnnn is calculated value. |

Notes to Table

- “Boot” commands are available on power-up or reset, Checksum will change when new application is downloaded,

- all the replies to host are given as they should be. The error replies are given in table 6

- In  NAOMI WFS, we are not using Utility board at all.

Table 5 SDSU VME APPLICATION Commands Defined for  WFS

| Command | Words Include header | Valid Destination DSPs | Response | Description |
| --- | --- | --- | --- | --- |
| ABT | 2 | VME ¦ TIMING | DON¦DAB (from VME) | AborT. Abort the previous RDC/RDS command. The host should send this command to the VME board only. |
| RDC | 2 | VME ¦ TIMING | DON (from VME) | ReaD CCD. Put the VEM DSP in readout mode using the current readout sequence and loaded parameters., wait for frame data from controller, The host should send this command to the VME board only. |
| RDS | 2 | VME ¦ TIMING | DON (from VME) | Same as RDC but data sent out to C40 through D24 port |
| FBA Aaaa,bbbb | 4 | VME | NONE | Aaaa,bbbb=Frame buffer address |

Notes to Table

- command in an application program which may be either downloaded or loaded from EEPROM using the LDA

- all the replies to host are given as they should be. The error replies are given in table 6

Table 6  SDSU Responses Defined for WFS

| Response | Description |
| --- | --- |
| DON | DONe. Command received and processed without error. |
| DAB | Done Abort during readout |
| SYR | System Reset. The controller has reset (power-on reset, reset button, RRS reset, or watchdog timer reset). |
|  | Error message |
| ERR | ERRor. Unrecognized command |
| POE | Power On Error. Error occurred while powering up the analog supplies on the utility board. |
| AFE | Address Format Error. A valid P, X, Y or EEPROM address was not specified. |
| HDE | HeaDer Error. The first word in the command has error |

# 11.  VME I/F board DSP codes for NAOMI WFS camera

The DSP code developed for NAOMI WFS camera consists of three parts:

1). Boot code 		vmeboot.asm

2). Head code		vmehead.asm

3). Application code 	vmeapl1.asm, vmeapl2.asm and vmeapl3.asm

vmeapl1.asm 	application 1

is used for reading out data to VxWork , the image can be displayed on SAOtng window and saved into a file.

vmeapl2.asm	application 2

is used for reading out data to C40.  No data can be seen by VxWork apart from frame state word (SFTAT)

vmeap3.asm	application 3

is used for self test for VME I/F board.  The simulated image can be displayed on SAOtng window and saved into a file.

Makefile

Three makefiles are available to use: (details seen Appendix)

1). Make -f makenorm.mk   vmebt_norn.s, vme_norm.s

compilers and links vmeboot and applications.  Later, *.s files are programmed into an E2prom (28c256) by a normal programmer.

2). Make -f makeevm56k.mk  vme56k.cld

compilers and links vmeboot and applications. Later, *.cld file is programmed into an E2prom (29c256, 27c256) by a motoroal 56k Evaluation Module board

3). Make -f makedownload.mk   vmeapl (1/2/3)_dwld.lod

compilers and links only applications.  Later, *.lod  files are downloaded directly into the DSP.

The DSP code flowchart is illustrated in Figure 7.

# 12. Modification to SDSU VME I/F board

It was discovered during the project that SDSU VME I/F board did not have a hardware means to detect whether a request of VMEbus interrupt has been granted. Various tests by a VME bus analyzer have illustrated that there could be a few microseconds delay for VME CPU board to respond to a VMEbus interrupt.  To make the system more reliable, a modification to SDSU VME I/F board has been made.  The wire links added to SDSU VME board are:

- wiring DSP portB.6  (DSP pin 15) to U33 pin 8 (internal connected to U22 pin5)

- for INT IRQ monitor

> wiring DSP portB.8  (DSP pin 7)  to parallel port pin 18 for FSYNC

> wiring DSP portB.10 (DSP pin 4)  to U30 pin 9 for FF

see the following diagram

DSP56002

U15

PB10

PB8

PB6

U30

Parallel port

D24

U22

U33

AUX 4 (pin 4)

AUX 2 (pin 7)

AUX 5 (pin 15)

FF (pin 9)

FSYNC (pin 18)

IRQ (pin 5)

IRQ (pin 8)

Figure 7 the VME I/F DSP code flowchart

# 13. CCD readout procedure ( for engineering level)

At engineering  level,  the system setup without C40 is shown in Figure 8

SDSU VME I/F  card

C40 convertor card

SDSU VME I/F  card

C40 convertor card

MVME167

MVME712

Work station

SDSU CCD controller

SDSU CCD controller

Ethernet

VME crate

straight RS232, port 1

CCD Header

CCD Header

Fiber cable

cable

Fiber cable

cable

Figure 8  the system setup without C40

Starting a CCD readout task involves three scripts at low level (VxWork):

1). Task

includes to startup Vxwork or Epics,then, startup sdsulib tasks, download DSP codes into VME I/F and Timing boards

2). Initial

setup parameters  for VME I/F ,Timing DSP code and  memory buffer in VME,

3). Read

start the CCD readout.

An example of CCD readout is of something like:

From a Xterm window,

type telnet naomi (could be another name),

type <startup.

Then type <startCCDReadout (see detail in Appendix )

There are few useful sdsuLib commands for engineering level ( details see sdsuLib manual ):

1). SdsuFrameShow "cam1" or  sdsuCurrentFrameShow "cam1" ( cam1 is the name for Master/slave )

display the frame status word and header information, the  address where the frame data is stored

2).sdsuDebug

if sdsuDebug=1, there will be printed message on screen when a frame interrupt occurs, set sdsuDebug=0, will disable the function.  It gives visible way to see if the frame readout is happening .

3). SdsuCommand ("cam1",1,"ABT")

this will abort the CCD readout task

4).  To read parameter

sdsuMemory (“cam1”,1,”RDM”,”CODE_ID”) or  sdsuCommand (“cam1”,2,”RDM”,0x******,0)

(where ****** is the address for P:10***, X: 20****, Y:40****,EEP:80****)

5). to write parameter

sdsuMemory (“cam1”,2,”WRM”,”EXP”) or  sdsuCommand (“cont1”,2,”WRM”,0x******,0)

(where ****** is the address for P:10***, X: 20****, Y:40****,EEP:80****)

# 14. Parameters  and memory map for VME I/F board DSP

Table 7  the  effects of different setting in parameters on VME DSP code

|  |  | Effects |  |
| --- | --- | --- | --- |
| DSPOCODE (in Y: memory) | Bits |  |  |
| WHICH | 0 | =0 donot send RDC to TIMing board, =1 send RDC to TIMing board, remove 12 garbage from frame data |  |
| RDHEAD | 1 | =0 donot check header, =1 check header |  |
| TWOPMD | 2 | =0 only one opmode in the header =1 two opmode in header |  |
| DOPIXC | 3 | =0 donot calculate PIXELC, use PIXEL_N set in initial by user =1 calculate PIXELC=rows*cols |  |
| TIMCK | 4 | =0 don't do time out for Half fifo full =1 do FO_TO time out check half fifi full |  |
| GARBAGE | 5 | =0 no garbage in data from Timing board code =1 remove NOFGAR garbage from frame data in Timing board code |  |
| CHKPXY (in X: memory) | Bits |  |  |
| CHKP | 0 | Do checksum for p memory ($0…$chk_sum) |  |
| CHKX | 1 | Do checksum for p, X memory ($0…$chk_sum, x) |  |
| CHKY | 2 | Do checksum for p,Y memory ($0…$chk_sum, y) |  |
| FSTAT_WIDTH |  | The frame start pulse for C40, unit=0.04us. default=2000 |  |
| READ_WIDTH |  | The READ FIFO pulse width for C40, unit=0.04us. default=5 |  |

Table 7  the Memory map for VME DSP code( all in HEX number)

|  | 56KVME memory location ( word wide ) |  |  | length(word) |  |  | EPROM memory Location (byte wide) |  |  | length | Difference in Words | DSP memory Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| progvme.asm | upwards | 7F00 |  |  |  |  |  |  |  |
| vmeboot.P | (BOOT_P) | 2000 | (BTLH_P) | 0300 | (EEPO_P) | C000 |  |  | P:RST_ISR |
| vmeboot.X | (BOOT_X) | 2300 | (BTLH_X) | 0100 | (EEPO_X) | C900 | 0900 |  | X:XPARAMT |
| vmeapl1.P | (APPL1_P) | 2400 | (APLH1_P) | 0200 | (EEAP1_P) | 8000 |  | 0000 | P:APL_ADR |
| vmeapl1.X | (APPL1_X) | 2600 | (APLH1_X) | 0100 | (EEAP1_X) | 8600 | 0600 | 0200 | X;USR_CMD |
| vmeapl1.Y | (APPL1_Y) | 2700 | (APLH1_Y) | 0100 | (EEAP1_Y) | 8900 | 0300 | 0300 | Y:0000 |
| vmeapl2.P | (APPL2_P) | 2800 | (APLH2_P) | 0200 | (EEAP2_P) | 8C00 | 0300 | 0400 | P:APL_ADR |
| vmeapl2.X | (APPL2_X) | 2A00 | (APLH2_X) | 0100 | (EEAP2_X) | 9200 | 0600 | 0600 | X;USR_CMD |
| vmeapl2.Y | (APPL2_Y) | 2B00 | (APLH2_Y) | 0100 | (EEAP2_Y) | 9500 | 0300 | 0700 | Y:0000 |
| vmeapl3.P | (APPL3_P) | 2C00 | (APLH3_P) | 0200 | (EEAP3_P) | 9800 | 0300 | 0800 | P:APL_ADR |
| vmeapl3.X | (APPL3_X) | 2E00 | (APLH3_X) | 0100 | (EEAP3_X) | 9E00 | 0600 | 0A00 | X;USR_CMD |
| vmeapl3.Y | (APPL3_Y) | 2F00 | (APLH3_Y) | 0100 | (EEAP3_Y) | A100 | 0300 | 0B00 | Y:0000 |

# 15. Check for code ID, author and version

Every VME DSP code has a version.  The CODE_ID and AUTHOR  are only in boot code.   Use sdsuMemory (“cam1”,1,”RDM”,”xxx”), where xxx is the symbol

| symbol | Expected reply | Version XX.XX | board XX | Or Application XX |
| --- | --- | --- | --- | --- |
| CODE_ID | "ATC" |  |  |  |
| AUTHOR | "GAO" |  |  |  |
| BT_CODE_VER | $010001 | 1.00 | 01 = VME Rev. 7A |  |
| APL_CODE_VER | $010001 | 1.00 |  | 01= application 1 |

# 16. Communications between SDSU controller Timing board and VME I/F board

Details for the communication between the VME board and the Timing board

ADC

AD0~AD15

D0~D23

AD0~AD23

VME DSP and D24 port

##### Timing DSP

SSI port

F.O transmitter

F.O. receiver

PAL: Shift register

FIFO : 8-bit

Each data (xx,xx,xx) has the form: 00, 01, AC (53), xx, xx, xx, 00

VME DSP sends CMDs through

PAL

Timing DSP

F.O transmitter

##### VME to Timing

##### Timing to VME

F.O receiver

PAL: shift register

FIFO: 16 bit

transceiver

Fiber 4Mhz

Decode if  preamble =$AC, then pass 24bit data to FIFO a byte at a time; if preamble =$53, reset DSP, others, ignore the data

1). WW=0, 24bit mode.  PAL expands the 24 bit data(xx,xx,xx) to AC,xx,xx,xx

2). WW=1,16bit  mode.  PAL transmits only low 16 bit of the 24 bit data from

DSP or 16 bit data from ADC

1). X:PBD.mode=0  shift register=16bit mode

2). X:PBD.mode=1  shift register=32bit mode

Fiber 50MHz

RDAD  dir: AD0~AD23 >> D0~D23

WRFO  dir: D0~D23      >>  AD0~AD23

require  400ns for 16bit data

The SDSU system processor block and communication path [3] . The boards in dash  line only

clock board

SSI serial

3.1Mbits/s [4]

Host computer

VME interface

Timing board

Utility board

VMEbus

D32,A32

Fiber optic

4Mbits/s

Fiber optic

50Mbits/s[4]

SSI serial

3.1Mbits/s [4]

DSP

DSP

DSP

PAL

PAL

Video board

or hardware decoded

for CCD operation reference.

# 17. Appendix

1). Make files

makeevm56k

#!/bin/csh

clear

echo  " This generates NAOMI VME DSP release version 1.00 boot code plus applications, "

echo  "       programed by 56kVME      ==>  vme56k.lst,.cld,vme56k.lod "

echo  "PRGAPL BOOT/APL1/APL2/APL3  program onlyboot/application1/2/3 "

echo  "PRG    EVM56K          program boot CODE for EVM56K  vmeboot_56k.lst"

echo  "PRG    EVM56K         program application CODE for EVM56K/"

echo  "                   vmeapl1_56k.lst  vmeapl2_56k.lst  vmeapl3_56k.lst  "

echo  "                  X.Gao           26-11-99      at  ATC   "

asm56000 -b -lvme56k.lst       -d PRGAPL APL1 -D PRGAPL2 APL2 -D PRGAPL3 APL3  progvme.asm

asm56000 -b -lvmeboot_56k.lst  -d PRG EVM56K   vmeboot.asm

asm56000 -b -lvmeapl1_56k.lst  -d PRG EVM56K  -d TEST HEAD -d READ MAUNAL vmeapl1.asm

asm56000 -b -lvmeapl2_56k.lst  -d PRG EVM56K  -d TEST HEAD -d READ MAUNAL vmeapl2.asm

asm56000 -b -lvmeapl3_56k.lst  -d PRG EVM56K  -d TEST HEAD -d READ MAUNAL vmeapl3.asm

dsplnk -bvme56k.cld  -v  progvme.cln vmeboot.cln vmeapl1.cln vmeapl2.cln vmeapl3.cln

rm     vme56k.lod

cldlod vme56k.cld >vme56k.lod

rm *.cln

makenorm

#!/bin/csh

clear

echo " This generates NAOMI VME DSP release code version 1.00 boot code plus applications,"

echo " programed by NORM programmer      vmebt_norm/vme_norm.lst,.cld,vmebt_norm/vme_norm.lod"

echo " PRGAPL BOOT/APL1/APL2/APL3  program onlyboot/application1/2/3 "

echo " PRG    NORM          program boot CODE for NORMAL EEPROM "

echo " PRG    NORM          program application CODE for NORMAL EEPROM "

echo "                  X.Gao           26-11-99      at  ATC"

echo "    BOOT first       ==> vmebt_norm.s"

asm56000 -b -lvmebt_norm.lst -d PRG NORM   vmeboot.asm

dsplnk 	 -bvmebt_norm.cld  -v  vmeboot.cln

rm        vmebt_norm.lod

cldlod    vmebt_norm.cld >vmebt_norm.lod

srec     -bs vmebt_norm.lod

echo  "   Now APPLICATIONS ==>  vme_norm.s"

asm56000 -b -lvmeapl1_norm.lst -d PRG NORM  -d TEST HEAD -d READ MAUNAL vmeapl1.asm

asm56000 -b -lvmeapl2_norm.lst -d PRG NORM  -d TEST HEAD -d READ MAUNAL vmeapl2.asm

asm56000 -b -lvmeapl3_norm.lst -d PRG NORM  -d TEST HEAD -d READ MAUNAL vmeapl3.asm

dsplnk -bvme_norm.cld  -v  vmeboot.cln vmeapl1.cln vmeapl2.cln vmeapl3.cln

rm     vme_norm.lod

cldlod vme_norm.cld >vme_norm.lod

srec -bs vme_norm.lod

rm   *.cln

makedowload

#!/bin/csh

clear

echo " build download version of VME application3 DSP code for VME board"

echo  "          <<<<< DOWNLOAD VERSION >>>>>    "

echo  "	vmeapl1_dwld.lod   vmeapl2_dwld.lod      vmeapl3_dwld.lod"

echo   "                X.Gao           26-11-99      at  ATC"

#vmeapl1_make

clear

echo  NAOMI VME application1 DSP code for VxWork, host sends RDC to VME

echo " <<<<< DOWNLOAD VERSION >>>>>     vmeapl1_dwld.lst, vmeapl1_dwld.lod "

echo "                  X.Gao           26-11-99      at  ATC          "

echo off

asm56000 -b -lvmebootdwld.lst -d PRG NORM   vmeboot.asm

asm56000 -b -lvmeapl1_dwld.lst -d PRG DOWNLOAD  -d TEST HEAD -d READ MAUNAL vmeapl1.asm

dsplnk -bvmeapl1_dwld.cld  -v  vmeapl1.cln vmeboot.cln

rm     vmeapl1_dwld.lod

cldlod vmeapl1_dwld.cld >vmeapl1_dwld.lod

#vmeapl2_make

clear

echo  NAOMI VME application2 DSP code for C40, host sends RDS to VME

echo  "<<<<< DOWNLOAD VERSION >>>>>     vmeapl2_dwld.lst, vmeapl2_dwld.lod"

echo  "                 X.Gao           26-11-99      at  ATC"

echo off

asm56000 -b -lvmebootdwld.lst -d PRG NORM   vmeboot.asm

asm56000 -b -lvmeapl2_dwld.lst -d PRG DOWNLOAD  -d TEST HEAD -d READ MAUNAL vmeapl2.asm

dsplnk -bvmeapl2_dwld.cld  -v  vmeapl2.cln vmeboot.cln

rm     vmeapl2_dwld.lod

cldlod vmeapl2_dwld.cld >vmeapl2_dwld.lod

#vmeapl3_make

clear

echo  NAOMI VME application3 DSP code for VME board selftest, host sends RDC to VME

echo  "<<<<< DOWNLOAD VERSION >>>>>     vmeapl3_dwld.lst, vmeapl3_dwld.lod"

echo   "                X.Gao           26-11-99      at  ATC"

echo off

asm56000 -b -lvmebootdwld.lst -d PRG NORM   vmeboot.asm

asm56000 -b -lvmeapl3_dwld.lst -d PRG DOWNLOAD  -d TEST HEAD -d READ MAUNAL vmeapl3.asm

dsplnk -bvmeapl3_dwld.cld  -v  vmeapl3.cln vmeboot.cln

rm     vmeapl3_dwld.lod

cldlod vmeapl3_dwld.cld >vmeapl3_dwld.lod

2). StartCCDReadout

# startup task

sdsuCommandStart ("cam1", 0xc0000000, 0)

sdsuMemoryStart ("cam1")

sdsuReadoutStart ("cam1")

sdsuMemory ("cam1", 1, "FDL", "/sw4/naomi/naomiWfs/dsp_code/xg/finalversion/vmeapl1_dwld.lod")

# initialise

sdsuCommand ("cam1", 2, "PON", 0, 0)

sdsuCommand ("cam1", 2, "SET", 100 , 0, 0)

sdsuMemory ("cam1", 1, "WRM", "DSPCODE", 46  )

sdsuMemory ("cam1", 1, "WRM", "PIXEL_N", 7040 )

sdsuMemory ("cam1", 1, "WRM", "FSTAT_WIDTH", 10)

sdsuMemory ("cam1", 1, "WRM", "NOFGAR", 8)

sdsuReadoutFrameQ ("cam1", 7040, 5)

sdsuCommand("cam1",2,"SYC",0,0)

# start readout of application 1

sdsuReadoutRDC("cam1", 50)

sdsuCommand("cam1",2,"LDA",1,0)

sdsuCommand("cam1",2,"SYC",0,0)

3). Default jump setting (most see SDSU VME Doc)

board address=0xc0000000 ( A31,A30 out, the rest in)

fiber optic speed =50MHz

address modifier=$

interrupt level=6  (IRQ6 in,  IACK0 in, others out)

bus request level =3

Eeprom =29c256( the same setting as 27c256), JP1 (1=2) JP2(1=2)

Watch dog timer = disabled

4). Example of test

login coll

startup another xterm

telnet naomi - can now communicate with VME rack

<startup

to display images

-----------------

startup another xterm

source /net/alba/sw4/naomi/naomilogin

naomiQI

a SAOTNG should become active

to start the VME self test readout process

----------------------------

naomiCam - cd "/sw4/naomi/naomiWfs/scripts/xg"

naomiCam -<xgVMEC40test

then you will see an image on SAOtng window.

naomiCam -sdsuDebug=1

will display the frame interrupt message, indicating the working condition

naomiCam -sdsuDebug=0

will stop

naomiCam - sdsuFrameShow "cam1"

Contents of SDSU frame located at  : 0xeb0000

Frame message queue     : 0xef3fc8

Frame semaphore ID      : 0xef7010

Frame header, located at address   : 0xeb0008

Packet count            : 209

Frame start count       : 0

VME board count         : 0

Timing board count      : 0x13b68

Frame status            : 0xc0

Operation mode          : 0x1

Exposure time           : 0xf4240

Number of rows          : 78

Number of columns       : 86

First 8 pixels, located at address : 0xeb0020

1234 5678 1357 2468 1234 5678 1357 2468

to start the VME/Timing self test readout process

----------------------------

<xgVME_cam1_dummy  ( for dummy star)

<xgVME_cam1_mode7 ( for simulated data)

to start the VME/Timing  readout process

----------------------------

<xgcam1Rd_Vx