- LIRIS PARK

AND

- WYFFOS DETECTOR

- LN2 AUTOMATIC FILLING SYSTEM

- ARM-TIME    MENU  OPTION

RMR  2012

ARM-TIME ?  MENU  OPTION

This option in the menu automatic filling system for Liris Park and Wyffos, selects the fill time between 1 and 99 hours.

- Normally this set is:

--- Liris Park: 24 h.

--- Wyffos: 8 h.

- To change the value:

--- Disarm the filling process. (NOT ARMED)

--- Select Menu. (MENU)

--- Select option ARM-TIME?

--- Set the value in hours.

--- Save the value.

--- Enable the filling process. (Re-ARMED)

- This option is useful if Liris cool head fails.

In this case we must change the Re-ARM value of 8 hours or less to maintain Liris cold.