From mpuig@ll.iac.esTue Apr 28 09:51:44 1998

Date: Mon, 27 Apr 1998 23:50:51 +0100 (WET DST)

From: Marti Pi i Puig <mpuig@ll.iac.es>

To: pcm@ing.iac.es

Cc: klt@ast.cam.ac.uk, map@ast.cam.ac.uk, mdcb@ing.iac.es, dji@ast.cam.ac.uk

Subject: Re: Ingrid Motors: epur si mouve

Hi Peter and all,

> from the oms and i/o card (are we still needing to use this ?)

I'm using this card *only* to detect driver fault (OMS cards does not

have inputs for this). If you want to remove the I/O card in the

future you'll need to remove or comment some parts of the code

and the database.

> Will these records be easy (:-)) to bring forward if we update EPICS

> versions, etc. ?

I think yes but I haven't tested in other versions. If not,

likely the records of the new version can be modified in the

same way.

> would like to come over and visit you for a day with Mathieu once he

> returns as he will be responsible for the mantanence of the EPICS sw here.

No problem I think. I'll prefer to have this talk at the IAC with the

hardware and to show you what I have now and how to modify it, if required.

Also, it can be a good moment for you to take away the hardware to La Palma.

This should be soon.

> Can you confirm that the software that you deliver is meant as the final

> version for integration to INGRID (* see below) ?

As you say, will be as ready as possible. Anyway, I haven't tested the

software with real mechanisms and that can trigger some litte problems.

Maybe speed/accl will be an important issue with the real

mechanisms. Also, now I'm using a 1:1 relation for the motor testrig

but wheels will have a 218:1 (in fact, this will improve resolution).

I'll put a 218:1 ratio in the final version.

Also, I'm making some assumptions. For example, I don't know the exact

positions in the wheels (in steps or degrees). For the filter wheels

that have 11 positions, I'm using  360/11=32.73... degrees between

positions (have a look at the attached file). This can be fine but

maybe needs to be adjusted with the real wheel.

So, maybe you'll need to edit some file.

Also, the software is not "ready to run". You'll need to install it,

configure it and rebuild it for your environment, VxWorks&EPICS versions,...

That always require some extra work.

> If we need to, could we expect that you would be able to travel to RGO,

> Cambridge, for a week when tesing takes place ?

I'm afraid no. Firsts tests were planned in March and then April. I didn't

received any new planning and I can't work more time on this.

It's a pitty because I've never been at the RGO...

Anyway, I'll help remotely by email as much as I can.

Also, if you think that the software have lots of bugs, or is a

f*ck*ng nightmare which never works fine, I'll would like to know it!

(excuse me for my language I'm just improving my english).

> Has Simon been able to talk with you about stuffing mechanism commands

> via CA to the database ? Have you agreed on names, conventions, etc.

We talk about it, and all is detailed at the ADD. The protocol is using

CAD/CAR/SIR EPICS records. If you want to change command names, filter

names, filter positions and so on you'll need to edit one of the

header files (imcs.h) which I'm attaching in this mail if you to

have a look and make some suggestions about names or messages.

> Name your prefered brand of whiskey !

Wait for the Ingrid "first light", and maybe then we can get drunk togheter :)

Cheers,

Marti

----------------------------------------------------------------------

MaRTi Pi i PuiG                            Phone : +34-(9)22-605-355

Instituto de Astrofisica de Canarias (IAC) Fax   : +34-(9)22-605-210

Software Engineering Department            Email : mpuig@iac.es

c/ Via Lactea s/n                                  martipi@computer.org

38200 La Laguna (Tenerife)                 Office: 1721

Canary Islands. SPAIN                      WWW   : http://www.iac.es