@0

new

@0

singleline(1)

separated(1)

channel = 1

direction = -1

max_steps = 3000

detent_width = 110

filter_space = 310

detent_mask = 0

datum_mask = 1

dim filter_pos[2]

@0

Proc zero1() {

set_up(1)

if(find_home())

print "0"

else

print "1"

}

@0

Proc zero2() {

set_up(2)

if(find_home())

print "0"

else

print "1"

}

@0

proc filter() {

set_up(%1)

b = %2

// determine direction in which to go

if (b != Filter_pos[channel]) {

if (b > Filter_pos[channel]) {

c = b - Filter_pos[channel] - 1

f =  max_steps * direction

} else {

c = Filter_pos[channel] - b - 1

f =  0

}

// start motion

move(channel, f)

// wait for this detent to release

while (moving(channel) && (de_bounce(detent_mask) == 0) {}

// count c detents

for (i = 0; i < c; i++) {

while (moving(channel) && (de_bounce(detent_mask) == 1) {}

while (moving(channel) && (de_bounce(detent_mask) == 0) {}

}

// capture required filter

if (find_detent()) {

filter_pos[channel] = b

print "0"

} else

print "1"

} else

print "0"

}

@0

func find_home() {

if (find_datum()) {

move(channel, filter_space * direction)

if (find_detent()) {

datum(channel)

filter_pos[channel] = 1

return 1

}

}

return 0

}

@0

func find_datum() {

// move away if we are at datum already

if (de_bounce(datum_mask) == 0) {

move(channel, where(channel) + (500 * direction))

while(moving(channel)){}

}

// move backwards full throw to datum park

move(channel, max_steps * direction * -1)

// sense datum and rough stop

while(moving(channel)) {

if(de_bounce(datum_mask) == 0) {

halt(channel)

while(moving(channel)) {}

return 1

}

}

return 0

}

@0

func find_detent() {

// make sure were moving

if (moving(channel) == 0)

return 0

// determine direction of motion

x1 = where(channel)

d = 0

while ((where(channel) == x) && moving(channel)) {}

if (((where(channel) - x) * direction) < 0)

d = 1

// make sure we are not in a datum hole

while(moving(channel) && (de_bounce(detent_mask) == 0)) {}

// setup for search

s = 1

x1 = 0

x2 = 0

// measure detent width

while(moving(channel)) {

if (de_bounce(detent_mask) != s) {

x = where(channel)

if (s) {

x1 = x

s = 0

} else {

x2 = x

halt(channel)

s = 1

}

}

}

// if we have approached backwards, go to first edge

if (d) {

move(channel, x1)

while(moving(channel)) {}

}

// step into calculated middle of detent

x = (x1 + x2) / 2

move(channel, x)

while(moving(channel)) {}

// check detent is home

if (de_bounce(detent_mask))

return 0

return 1

}

@0

proc setup() {

if (%1 < 2) {

// filter wheel 1

channel = 1

direction = 1

detent_mask = 0

datum_mask = 1

} else {

// filter wheel 2

channel = 0

direction = -1

detent_mask = 2

datum_mask = 3

}

}

@0

func de_bounce() {

// simple majority vote on three

q = 0

for (i = 0; i < 3; i++) {

q += in(%1)

wait(5)

}

if (q > 1)

return 1

return 0

}

@1

new

@1

singleline(1)

separated(1)

proc zerofoc() {

//move away from datum if already in region

if (in(4)==0){

move(0,1800)

while(moving()){}

}

move(0,-5500)

while(moving()){

if (in(4)==0){

halt(0)

x=where(0)

for(i=1;in(4)==0;i++){

move(0,x+i)

while(moving()){}

}

print"0"

datum(1)

}

}

}

@1

proc focus(){

move(0,%1)

}