ing technical specification

data acquisition system interface

version 00.00			date 970801		author

1. introduction.

Page	1	1.0 Purpose of document.

1	1.1 Document version control.

1	1.2 Scope of the document.

1	1.3 Who should read it and use it.

2. technical requirements of interface.

Page	1	2.0 Logical requirements.

1	2.1 Functional requirements.

1	2.2 Physical requirements.

1	2.3  Capacity requirements

3. description of system elements.

Page	1	3.0 Hardware architecture of data acquisition system.

1	3.1 Hardware architecture of detector controllers.

1	3.2 System topologies.

1	3.3 Software architecture of data acquisition system.

1	3.4 Software architecture of detector controller.

4. interface protocol description.

Page	1	4.0 Command / acknowledge channels.

1	4.1 Image data channel.

1	4.1 Message (Status and Engineering) channel.

1	4.2 Syncronization and time stamp channel.

5. description of interface ports.

Page	1	5.0 Command port.

1	5.1 Data port..

1	5.2 Message (Status and Engineering) port.

1	5.3 Time stamp port.

1	5.4 Synchronization port.

6. electrical / optical standards description.

7. mechanical standards description.

Page	1	7.0 Fiber optic port connectors and cables.

1	7.1 RS232 port connectors and cables.

1	7.2 Power connectors and cables.

1	7.3 Time stamp port connectors and cables.

1	7.4 Synchronization port connectors and cables.

8.  sdsu controller example.

Page	1	8.0 Timing board command structure.

1	8.1 Utility board command structure.

1. INTRODUCTION

1.0 Purpose of document.

This document defines a logical, electrical, and functional interface standard for the ING telescopes that enables the seamless integration of detector controller subsystems to the common telescope data acquisition system (DJ). By establishing this interface, all developers of detector controller (CCDC) hardware and software who wish to use the ING DJ system are presented with a clear and concise definition of the capabilities of the system  and the methods of using these capabilities.

1.1 Document version control.

The responsibility of document control is placed with the ING Software Group who are responsible for the strict change and version control of it. The generalized aim of version control is to assure that increasing minor version number changes will assure compatibility to all lower versions with the same major version number. The major version number will be used to designate a generalized area of use (e.g. autoguiders, science ccd’s, photon counters, etc.) or be used to indicate incompatibility with prior releases when compatibility cannot be achieved. It is recommended that the major version numbers between 90 and 99 be reserved for development and experimental use only. It would be most productive if version number compliance can be indicated in CCDC and DJ software version numbering. The ING Software Group also have the responsibilty to provide interpretation of the document contents.

1.2 Scope of the document.

1.3 Who should read it and use it.

Anyone involved with the development and use of detector controllers for use on instrumentation supported by the ING. The ING provides a large portion of the infrastructure required to support observations on the telescopes, therefore, anyone who wants to make their instrument work reliably within that infrastructure is required to read and understand this document. Please refer to the ING Software Group for an interpretation of any specific wording in this document.

2. technical requirements of interface.

2.0 Logical requirements.

Detector controller identification

physical type of controller

type of service

addressing and selection of controller

scope and visibility of address, selection, identification

Detector identification

physical detector type

detector readout mode (sequencer code) requirements

detector enviroment (bias voltages, temperature, etc.) requirements

Image data channel identification

storage requirements

post acquisition processing requiements

2.1 Functional requirements.

The acquisition of an astronomical image involves the following processes:-

detector stabalization

detector reset

shutter control

image integration timing

detector readout

analog to digital conversion

image data transfer

post acquisition processes on image data

collection of image descriptors

formatting of image data and descriptors

storage of formatted image file

serving of image file to requesting clients

error recovery

syncronization of acquisition processes

The functional requirements and the distribution of the processes involved in acquiring an electronic image and storing it with sufficient detail of how it was acquired can be clasified in the following way.

| functional clasification | Data Acquisition processes | Sub system responsible |
| --- | --- | --- |
| detector control | detector stabalization | detector controller |
|  | detector reset | detector controller |
|  | detector readout | detector controller |
|  | analog to digital conversion | detector controller |
| image integration control | shutter control | detector controller |
|  | integration timing | detector controller |
|  | preflash timing | detector controller |
| image data transfer control | image data transmission | detector controller |
|  | image data reception | Data Junction |
|  | image data error checking | Data Junction |
| post acquisition processing | image stitching | Data Junction |
|  | image data normalization | Data Junction |
|  | image data stacking | Data Junction |
| image descriptor collection | detector descriptors | Data Junction |
|  | timing descriptors | Data Junction |
|  | instrument descriptors | Data Junction |
|  | telescope descriptors | Data Junction |
|  | misc descriptors (eg. meteology) | Data Junction |
| format and storage control | formatting | Data Junction |
|  | storage | Data Junction |
| image serving | client request processing | Data Junction |
|  | image transmission | Data Junction |
| error recovery | recovery of control | Data Junction |
|  | recovery of data | Data Junction |
| syncronization control | state mapping | Data Junction |
|  | status collection | Data Junction |

2.2 Physical requirements.

2.3 Capacity requirements.