Operation/ testing procedures for  Naomi WFS CCD camera

Naomi WFS camera has two EEV CCD39-01 DUAL in-line Package with integral2-stage Peltier cooler.

The CCD39-01 is back illuminated with 80*80(plus 4 underscan each row) pixels, each pixel has 24 um square.

The CCD is cooled by means of supplying DC current to the package.  At present, the DC current is generated by DC voltage supplier with serial resistor ( see Brenda ‘s Docs).

The camera system layout is illustrated in Figure 1

MVME167

Electra

Interface

Epics / vxWorks

SDSULib

SDSU VME I/F  card

SDSU CCD controller

C40 convertor card

C40 Interface

C40 Array

SDSU VME I/F  card

SDSU CCD controller

C40 convertor card

C40 Interface

Commands

data

data

Water Cooler set to 8C

Flow sensor

DC power supplier with interlock by flow sensor

Heat exchanger

Heat exchanger

CCD 39 Peltier package

CCD 39 Peltier package

Heat exchanger

Heat exchanger

Figure 1. the Naomi WFS camera system layout

To operate the camera, test CCD:

1). Switch on the Water cooler first

2). Switch on VME crate, press the reset button of SDSU VME I/F boards

3). Switch on the SDSU controller

4). Switch on Peltier power suppler

5). Login  Navis, from  one xterm, type

source /software/naomi/naomi-0.2/config/setup.csh

source /software/naomi_atc_src/naomiView/naomiView_login

naomiQI_xg

6) open another xterm rlogin ( or telnet)  naomisdsu , this gets you into the Vxwork shell

7). To start full frame ccd readout as default using Vxwork, QI-Sever, SAOtng to display image, from Vxwork shell

<readout_cam1  or  <close1 to stop readout

<readout_cam2 or  <close2 to stop readout

- cam1 refer to Master, can2 refer to Slave.  During the unsynchronised readout mode, you have to stop cam1 before  to start cam2, vice verca

8). To change parameters, like exposure time, speed. etc, see Derek’s ICD .

9). To make change and recompile, see READMEfirst.doc

10). To do noise, dark current measurement, see Derek’s note  Camera_result.doc

11). To use C40 system, see Durham’s Docs.

12). To move camera, etc,  see other Docs , like Chris Tierney’s.

13). Read READMEfirst.doc for more details