global memory address decode scheme

| simm organisation | simm sites | memory size | address range | strb0 pagesize | strb1 pagesize |  | strbactive |  | pd1 | pd2 | sw0 | sw1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 x 32 MBit | J7, 8, 9, 10 | 128 Mbyte ! | (80)000000 - (81)ffffff | 10101 |  | 10101 |  | 10111 | 1 | 0 | 1 | 1 |
| 3 x 32 MBit | J7, 8, 9 | 96 Mbyte ! | (80)000000 - (81)7fffff | 10101 |  | 10101 |  | 10111 | 1 | 0 | 0 | 1 |
| 2 x 32 MBit | J7, 8 | 64 Mbyte ! | (80)000000 - (80)ffffff | 10101 |  | 00111 |  | 10111 | 1 | 0 | 1 | 0 |
| 1 x 32 MBit | J7 | 32 Mbyte ! | (80)000000 - (80)7fffff | 10101 |  | 00111 |  | 10111 | 1 | 0 | 0 | 0 |
| 4 x 16 MBit | J7, 8, 9, 10 | 60 Mbyte * | (80)100000 - (80)ffffff | 01000 |  | 10101 |  | 10110 | 0 | 1 | 1 | 1 |
| 3 x 16 MBit | J7, 8, 9 | 44 Mbyte * | (80)100000 - (80)bfffff | 01000 |  | 10101 |  | 10110 | 0 | 1 | 0 | 1 |
| 2 x 16 MBit | J7, 8 | 28 Mbyte * | (80)100000 - (80)7fffff | 01000 |  | 10101 |  | 10110 | 0 | 1 | 1 | 0 |
| 1 x 16 MBit | J7 | 12 Mbyte * | (80)100000 - (80)3fffff | 01000 |  | 10101 |  | 10110 | 0 | 1 | 0 | 0 |
| 4 x 8 MBit | J7, 8, 9, 10 | 32 MByte | (80)100000 - (80)8fffff | 01000 |  | 10011 |  | 10011 | 1 | 1 | 1 | 1 |
| 3 x 8 MBit | J7, 8, 9 | 24 MByte | (80)100000 - (80)6fffff | 01000 |  | 10011 |  | 10101 | 1 | 1 | 0 | 1 |
| 2 x 8 MBit | J7, 8 | 16 MByte | (80)100000 - (80)4fffff | 01000 |  | 10011 |  | 10101 | 1 | 1 | 1 | 0 |
| 1 x 8 MBit | J7 | 8 MByte | (80)100000 - (80)2fffff | 01000 |  | 10011 |  | 10101 | 1 | 1 | 0 | 0 |

NOTES:

! A24 must be set high through the smt307 global expansion register to enable global memory,

boot code exists in flash ram between addresses 8000000 - 8007FFFF which is enabled at reset.

* 4Mbytes of ram memory is shadowed by the boot flash rom on strobe 0

row address decode scheme

| pd1 | pd2 | sw0 | sw1 | ras00 ras02 | ras01 ras03 | ras10 ras12 | ras11 ras13 | ras20 ras22 | ras21 ras23 | ras30 ras32 | ras31 ras33 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 80000000 803fffff | 80400000 807fffff | 80800000 80bfffff | 80c00000 80ffffff | 81000000 813fffff | 81400000 817fffff | 81800000 81bfffff | 81c00000 81ffffff |  |  |  |
| 1 | 0 | 0 | 1 | 80000000 803fffff | 80400000 807fffff | 80800000 80bfffff | 80c00000 80ffffff | 81000000 813fffff | 81400000 817fffff |  |  |  |  |  |
| 1 | 0 | 1 | 0 | 80000000 803fffff | 80400000 807fffff | 80800000 80bfffff | 80c00000 80ffffff |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 80000000 803fffff | 80400000 807fffff |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 1 | 80100000 803fffff |  | 80400000 807fffff |  | 80800000 80bfffff |  | 80c00000 80ffffff |  |  |  |  |
| 0 | 1 | 0 | 1 | 80100000 803fffff |  | 80400000 807fffff |  | 80800000 80bfffff |  |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 80100000 803fffff |  | 80400000 807fffff |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 | 0 | 80100000 803fffff |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 80100000 801fffff | 80200000 802fffff | 80300000 803fffff | 80400000 804fffff | 80500000 805fffff | 80600000 807fffff | 80700000 807fffff | 80800000 808fffff |  |  |  |
| 1 | 1 | 0 | 1 | 80100000 801fffff | 80200000 802fffff | 80300000 803fffff | 80400000 804fffff | 80500000 805fffff | 80600000 807fffff |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 80100000 801fffff | 80200000 802fffff | 80300000 803fffff | 80400000 804fffff |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | 80100000 801fffff | 80200000 802fffff |  |  |  |  |  |  |  |  |  |

ROW Address decode truth table

| logic inputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | logic outputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| size0 | size1 | /refresh | /ras |  | stat3 |  | a20 |  | a21 | a22 |  | a23 |  |  |  | /ras00 |  |  | /ras01 |  |  | /ras10 |  | /ras11 |  | /ras20 |  | /ras21 |  | /ras30 |  | /ras31 |  |  |  |
| x | x | x | 1 |  | x |  | x |  | x | x |  | x |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| x | x | 0 | 0 |  | x |  | x |  | x | x |  | x |  |  |  | 0 |  |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  |
| 32MBYTE DEVICE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 |  | 0 |  | x |  | x | 0 |  | 0 |  |  |  | 0 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 0 |  | x |  | x | 1 |  | 0 |  |  |  | 1 |  |  | 0 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 0 |  | x |  | x | 0 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 0 |  | x |  | x | 1 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 1 |  | x |  | x | 0 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 1 |  | x |  | x | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 1 |  | x |  | x | 0 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  | 1 |  | x |  | x | 1 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  |  |  |
| 16MBYTE DEVICE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 0 |  | 1 |  | x |  | x | 0 |  | 0 |  |  |  | 0 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 0 | 1 | 1 | 0 |  | 1 |  | x |  | x | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 0 | 1 | 1 | 0 |  | 1 |  | x |  | x | 0 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  |
| 0 | 1 | 1 | 0 |  | 1 |  | x |  | x | 1 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  |
| 8MBYTE DEVICE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 1 |  | 0 | 0 |  | 0 |  |  |  | 0 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 0 |  | 1 | 0 |  | 0 |  |  |  | 1 |  |  | 0 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 1 |  | 1 | 0 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 0 |  | 0 | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 1 |  | 0 | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 0 |  | 1 | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 1 |  | 1 | 1 |  | 0 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  | 1 |  | 0 |  | 0 | 0 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  |  |  |

ROW ADDRESS Decode logic algorithm

| SIGNAL | DECODE |
| --- | --- |
| /RAS00 | /RAS * PD1 * /PD2 * /STAT3 * /A22 * /A23 + /RAS * /PD1 * PD2 * STAT3 * /A22 * /A23 + /RAS * PD1 * PD2 * STAT3 * A20 * /A21 * /A22 * /A23 + /REFRESH |
| /RAS01 | /RAS * PD1 * /PD2 * /STAT3 * A22 * /A23 + /RAS * PD1 * PD2 * STAT3 * /A20 * A21 * /A22 * /A23 + /REFRESH |
| /RAS10 | /RAS * PD1 * /PD2 * /STAT3 * /A22 * A23 + /RAS * /PD1 * PD2 * STAT3 * A22 * /A23 + /RAS * PD1 * PD2 * STAT3 * A20 * A21 * /A22 * /A23 + /REFRESH |
| /RAS11 | /RAS * PD1 * /PD2 * /STAT3 * A22 * A23 + /RAS * PD1 * PD2 * STAT3 * /A20 * /A21 * A22 * /A23 + /REFRESH |
| /RAS20 | /RAS * PD1 * /PD2 * STAT3 * /A22 * /A23 + /RAS * /PD1 * PD2 * STAT3 * /A22 * A23 + /RAS * PD1 * PD2 * STAT3 * A20 * /A21 * A22 * /A23 + /REFRESH |
| /RAS21 | /RAS * PD1 * /PD2 * STAT3 * A22 * /A23 + /RAS * PD1 * PD2 * STAT3 * /A20 * A21 * A22 * /A23 + /REFRESH |
| /RAS30 | /RAS * PD1 * /PD2 * STAT3 * /A22 * A23 + /RAS * /PD1 * PD2 * STAT3 * A22 * A23 + /RAS * PD1 * PD2 * STAT3 * A20 * A21 * A22 * /A23 + /REFRESH |
| /RAS31 | /RAS * PD1 * /PD2 * STAT3 * A22 * A23 + /RAS * PD1 * PD2 * STAT3 * /A20 * /A21 * /A22 * A23 + /REFRESH |

MEMORY ADDRESS MUX CODING

|  | SHORTRAS |  |  | LONGRAS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| addr |  | ras | cas |  | ras | cas |
| MA00 |  | A10 | A00 |  | A11 | A00 |
| MA01 |  | A11 | A01 |  | A12 | A01 |
| MA02 |  | A12 | A02 |  | A13 | A02 |
| MA03 |  | A13 | A03 |  | A14 | A03 |
| MA04 |  | A14 | A04 |  | A15 | A04 |
| MA05 |  | A15 | A05 |  | A16 | A05 |
| MA06 |  | A16 | A06 |  | A17 | A06 |
| MA07 |  | A17 | A07 |  | A18 | A07 |
| MA08 |  | A18 | A08 |  | A19 | A08 |
| MA09 |  | A19 | A09 |  | A20 | A09 |
| MA10 |  | A20 | A10 |  | A21 | A10 |
| MA11 |  | A21 | A11 |  | A22 | A11 |

U2 PIN ASSIGNMENTS (PALCE20V10) -- ROW ADDRESS DECODE PAL

| PIN # | DES | SIG | I/O | NOTES |
| --- | --- | --- | --- | --- |
|  | I0 | GAD20 | I | ADDRESS 20 |
|  | I1 | GAD21 | I | ADDRESS 21 |
|  | I2 | GAD22 | I | ADDRESS 22 |
|  | I3 | GAD23 | I | ADDRESS 23 |
|  | I4 | STAT3 | I | STROBE 0/1 |
|  | I5 |  |  | NOT CONNECTED |
|  | I6 | /REFRESH | I | FROM U6 |
|  | I7 | /RAS | I | FROM U6 |
|  | I8 | /SPARE | I | FROM U6 |
|  | I9 | /SPARE | I | FROM U6 |
|  | I/O0 | /RAS00 | O | J7 LOWER ROW SELECT |
|  | I/O1 | /RAS01 | O | J7 UPPER ROW SELECT |
|  | I/O2 | /RAS10 | O | J8 LOWER ROW SELECT |
|  | I/O3 | /RAS11 | O | J8 UPPER ROW SELECT |
|  | I/O4 | /RAS20 | O | J9 LOWER ROW SELECT |
|  | I/O5 | /RAS21 | O | J9 UPPER ROW SELECT |
|  | I/O6 | /RAS30 | O | J10 LOWER ROW SELECT |
|  | I/O7 | /RAS31 | O | J10 UPPER ROW SELECT |
|  | I/O8 | PD1 | I | SIZE0 ARG: - 32MB=1, 16MB=0, 8MB=1 |
|  | I/O9 | PD2 | I | SIZE1 ARG: - 32MB=0, 16MB=1, 8MB=1 |

U6 PIN DEFINITIONS (MACH 210) -- MEMORY TIMING CONTROLLER

| PIN # | DES | SIG | I/O | NOTES |
| --- | --- | --- | --- | --- |
|  | IN0 | 50CLK | I | 50 Mhz BASIC CLOCK |
|  | IN1 | /STRB0 | I | TIM40 MEMORY BANK 0 STROBE |
|  | IN2 | /STRB1 | I | TIM40 MEMORY BANK 1 STROBE |
|  | IN3 | H3CLK | I | TIM40 25Mhz CLOCK |
|  | IN4 |  | I |  |
|  | IN5 |  | I |  |
|  | I/O0 | PAGE0 | I |  |
|  | I/O1 | RNWD0 | I |  |
|  | I/O2 | RDY0 | O |  |
|  | I/O3 | PAG1 | I |  |
|  | I/O4 | RNWD1 | I |  |
|  | I/O5 | RDY1 | O |  |
|  | I/O6 |  |  |  |
|  | I/O7 |  |  |  |
|  | I/O8 | /CONFIG | I |  |
|  | I/O9 | /RESET | O | RESET GENERATOR OUTPUT |
|  | I/O10 | /WE | O | WRITE ENABLE TO RAM BANKS |
|  | I/O11 | /CAS0 | O | COMMON CAS STROBE TO RAM BANKS |
|  | I/O12 | /CAS1 | O | COMMON CAS STROBE TO RAM BANKS |
|  | I/O13 | STAT3 | I | TIM40 MEMORY ACCESS STATUS NIBBLE |
|  | I/O14 | STAT2 | I |  |
|  | I/O15 | STAT1 | I |  |
|  | I/O16 | STAT0 | I |  |
|  | I/O17 | LOCK | I | TIM40 MEMORY LOCK, USED TO DETERMINE REFRESH CYCLE |
|  | I/O18 | /REFRESH | O |  |
|  | I/O19 | /RAS | O |  |
|  | I/O20 | /SPARE | O |  |
|  | I/O21 | /SPARE | O |  |
|  | I/O22 | PD4 | I |  |
|  | I/O23 | PD3 | I |  |
|  | I/O24 | PD2 | I |  |
|  | I/O25 | PD1 | I |  |
|  | I/O26 | /STATUS | O |  |
|  | I/O27 |  |  |  |
|  | I/O28 | ENBL0 | O | CAS ADDRESS ENABLE |
|  | I/O29 | ENBL1 | O | 8MBIT RAS ADDRESS ENABLE |
|  | I/O30 | ENBL2 | O | 32MBIT RAS ADDRESS ENABLE |
|  | I/O31 | ENBL3 | O | 64MBIT RAS ADDRESS ENABLE |