Mechanical Work Package request

| Project code | GLAS |
| --- | --- |

To be filled in by the person requesting the work

| Date | 18 February 2008 |
| --- | --- |
| Demanding officer | Jure Skvarč |
| Description work required (design/ manufacture) | Create a mount for the new Andor camera to replace AG7. The mount should allow movement in x, y and z by +-5 mm and a small amount of rotation (+-3 degrees) must be possible to precisely align the camera. The mount must be easy to replace with existing mount in case if we fall back to AG7. Mechanical knobs must allow repeatable alignment within 10 micrometers. Nominal position of the camera when mounted with the knobs in the middle position is in the focal point of the “simplex” camera with centre of the CCD on the optical axis. Work to be done: - Visual inspection of the existing setup in GRACE and checking for any obstacles. Together with the client. - Creation of drawings which will have to be approved by the client. - Construction of the mount. - Mounting of the new mount together with the camera and rough alignment. The camera focal plane needs to be at 350mm from the simplexing flat mirror The act of mount and camera replacement might be postponed to a later date for operational reasons. drawing number. |
| Date required for completion | 28 April 2008 |
| Interfaces to be satisfied by completed product |  |
| Interfaces to be maintained during the work |  |
| Any constraints to be observed | All electrical and cooling connectors must have sufficient clearing so that the connectors can be attached when camera is mounted. |

| Joint agreement on effort/cost/time | Demanding officer |  |
| --- | --- | --- |
|  | Deputy Head of Operations |  |

To be completed by the workshop manager

| Work package acceptance workshop manager |  |
| --- | --- |
| Completion confirmation workshop manager |  |
| Agreed hand over work package |  |